SIMATS ENGINEERING

ASSESSMENT TOOL 3: Mock Interview

COURSE NAME: COMPUTER VISION	COURSE CODE: ITA0515	Slot B

COURSE OUTCOME COVERED

CO5: Develop computer vision applications using OpenCV for performing recognition and analysis on images and videos. (BTL 6)

Assessment Tool Weightage: 10%

QUESTIONS:4	Total Marks 20	Duration :60 Minutes Annexure A

Mock Interview

Code of Conduct:

I K Sreelatha(192425248)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. IL= understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Sree K

Rubrics for Evaluation

| Criteria | Weightag e (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing ) | Level 1 (Beginning) | Marks(100 ) |
| --- | --- | --- | --- | --- | --- | --- |
| Technical Knowledge | 25 | Demonstrates strong and accurate subject knowledge with in-depth understanding | Shows good knowledge with minor gaps | Basic knowledge with noticeable errors | Lacks fundamental technical knowledge |  |
| Problem Solving | 25 | Solves problems logically and applies concepts effectively in real scenarios | Applies concepts with moderate accuracy | Limited problem-solving ability; requires guidance | Unable to solve or apply concepts |  |
| Analytical Thinking | 15 | Analyzes questions critically and provides well-structured, logical answers | Provides reasonable analysis with some structure | Superficial or partially structured responses | No analytical thinking demonstrated |  |
| Communication | 15 | Communicates clearly, confidently, and professionally with proper terminology | Communicates well with minor hesitation | Communication lacks clarity or confidence | Unable to communicate effectively |  |
| Confidence | 15 | Displays high confidence, positive attitude, and professional behavior throughout | Shows good confidence with minor issues | Low confidence; inconsistent behavior | Lacks confidence and professionalism |  |
| Response to Questions | 10 | Responds accurately, promptly, and elaborates with relevant examples | Responds correctly but with limited depth | Partial responses; needs prompting | Unable to respond appropriately |  |

OpenCV Basics & Pipeline Thinking

- Design and explain a complete OpenCV pipeline to read, process, and display multiple images from a folder efficiently. Clearly justify your design choices and discuss potential challenges and optimizations.

- A video file is not loading properly in OpenCV. Analyze the possible causes and propose a structured debugging approach, explaining each step logically.

- You need to overlay text and shapes on a live video feed. Design your solution and explain how you would implement it efficiently, including key OpenCV functions and reasoning.

- An image appears too dark when read. Analyze the issue and propose modifications to your processing pipeline, justifying how your approach improves visibility.

- OpenCV Pipeline for Processing Multiple Images from a Folder Pipeline

Folder → Read Images → Validate → Preprocess → Process → Draw/Annotate → Display/Save

Design

  - Get image filenames

    - Use Python os or glob to collect image paths from the folder.

    - Filter only supported formats such as .jpg, .jpeg, and .png.

# Read images

    - Use cv2.imread() to load each image.

    - Check whether the returned image is None to identify corrupted or unsupported files.

# Preprocessing

    - Resize images to a common size using cv2.resize().

    - Convert to grayscale using cv2.cvtColor() if color information is not required.

    - Apply Gaussian or median filtering when noise is present.

# Processing

    - Perform the required operation such as edge detection, thresholding, feature extraction, or object detection.

# Display

    - Use cv2.imshow() to display each processed image.

    - Use cv2.waitKey() to control the display time.

# Save Results

    - Use cv2.imwrite() to save processed images in a separate output folder.

# Efficiency and Optimization

- Resize large images before processing to reduce CPU usage.

- Process images one at a time instead of loading the entire folder into memory.

- Use batch processing when the number of images is large.

- Avoid repeated conversions and unnecessary copies.

- Use parallel processing for independent images when the dataset is very large.

# Challenges

Different image sizes, corrupted files, unsupported formats, high memory usage, and large processing time can affect the system. Proper validation, resizing, error handling, and efficient memory management make the pipeline reliable.

# Debugging a Video File That Does Not Load in OpenCV

A structured debugging process should be followed rather than immediately changing the code.

# Step 1: Check the File Path

Verify that the video exists at the specified location. cap = cv2.VideoCapture("video.mp4")

A wrong path or filename is one of the most common causes.

# Step 2: Check Whether the Video Opened

if not cap.isOpened(): print("Unable to open video")

If this fails, the problem is usually related to the file, codec, path, or OpenCV installation.

# Step 3: Check the Video Properties

Read properties such as:

- Frame width

- Frame height

- FPS

- Number of frames

fps = cap.get(cv2.CAP_PROP_FPS)

width = cap.get(cv2.CAP_PROP_FRAME_WIDTH) height = cap.get(cv2.CAP_PROP_FRAME_HEIGHT) Invalid values may indicate a codec or file problem.

# Step 4: Try Reading a Frame

ret, frame = cap.read()

if not ret:

print("Frame could not be read")

If the video opens but frames cannot be read, the codec or video file may be problematic.

# Step 5: Check Codec Compatibility

Some video formats use codecs that are not supported by the installed OpenCV/FFmpeg backend. Try converting the video to a commonly supported format such as MP4 with H.264.

# Step 6: Test Another Video

Try loading a known-good video. If it works, the original video is likely corrupted or encoded with an unsupported codec.

# Step 7: Check Installation

Verify that OpenCV has appropriate video backend support. Reinstalling OpenCV or using a build with FFmpeg support may solve the issue.

# Conclusion

The debugging sequence should be:

Path → isOpened() → Video properties → read() → Codec → Test another video → Check OpenCV/FFmpeg

This systematic approach identifies whether the problem is caused by the file, path, codec, or software environment.

# Overlaying Text and Shapes on a Live Video Feed Pipeline

Camera → Capture Frame → Draw Shapes/Text → Display → Repeat Implementation

  - Open the camera:

cap = cv2.VideoCapture(0)

  - Read frames continuously: ret, frame = cap.read()

  - Draw shapes using OpenCV functions:

- cv2.rectangle() – bounding boxes

- cv2.circle() – circular markers

- cv2.line() – lines

- cv2.polylines() – polygons

  - Add text using: cv2.putText(frame, "Object Detected",

(50,50), cv2.FONT_HERSHEY_SIMPLEX, 1, (255,255,255), 2)

  - Display the modified frame: cv2.imshow("Live Feed", frame)

  - Exit when the user presses a specified key.

# Efficiency

- Keep the camera resolution appropriate for the application.

- Draw only the required objects.

- Avoid unnecessary image copies.

- Process only the Region of Interest when possible.

- Use simple drawing operations because they have low computational cost.

# Example Flow

Camera

↓

Capture Frame

↓

Detect/Analyze Objects

↓

Draw Rectangle/Circle/Line

↓

Add Text and Labels

↓

Display Live Frame

↓

Next Frame

# Conclusion

OpenCV drawing functions can directly modify each captured frame, making them suitable for real-time overlays such as object labels, FPS counters, bounding boxes, and status information.

# Image Appears Too Dark When Read

A dark image may result from poor lighting, incorrect exposure, low contrast, or an unsuitable processing step. The problem should first be analyzed before applying enhancement.

# Step 1: Verify Image Reading

Check whether the image was successfully loaded:

img = cv2.imread("image.jpg")

if img is None: print("Image not loaded")

# Step 2: Check Color Representation

OpenCV reads color images in BGR format, not RGB. If color conversion is required, use: rgb = cv2.cvtColor(img, cv2.COLOR_BGR2RGB)

# Step 3: Improve Brightness

Brightness can be increased by adjusting pixel intensity: Enhanced = Image + Brightness Value

Care should be taken to avoid overexposure.

# Step 4: Histogram Equalization

Convert the image to grayscale and apply:

gray = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY) enhanced = cv2.equalizeHist(gray)

This improves global contrast and makes dark regions more visible.

# Step 5: Use CLAHE

For images with uneven illumination, CLAHE is often better:

clahe = cv2.createCLAHE() enhanced = clahe.apply(gray)

It enhances local contrast while limiting excessive amplification.

# Step 6: Gamma Correction

Gamma correction can brighten dark images while preserving useful details: Output = Input^γ

Using a suitable gamma value below 1 generally brightens a dark image.

# Conclusion

A suitable pipeline is:

Read Image → Validate → Convert Color/Grayscale → Brightness or Gamma Correction → CLAHE/Histogram Equalization → Display

CLAHE and gamma correction are particularly useful when the image contains important details hidden in dark regions.