SIMATS ENGINEERING

ASSESSMENT TOOL 2: Case Study

COURSE NAME: COMPUTER VISION		  COURSE CODE:ITA0515          Slot B

COURSE OUTCOME COVERED

CO3: Analyze and extract image features using detection and matching techniques for effective image representation and comparison. (BTL 4)

Assessment Tool Weightage: 40%

QUESTIONS:5				Total Marks 50                                 Duration :60 Minutes

Annexure A

Case Study

Code of Conduct:

I K Sreelatha(192425248) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:Sree K

Rubrics for Evaluation

| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) | Marks(100) |
| --- | --- | --- | --- | --- | --- | --- |
| Understanding of Case | 25 | Thorough understanding; clearly identifies all key issues | Good understanding with most issues identified | Basic understanding; some issues missed | Poor understanding; problem not identified correctly |  |
| Application of Concepts | 25 | Effectively applies relevant concepts to analyze the case deeply | Applies appropriate concepts with minor gaps | Limited application of concepts | Fails to apply relevant concepts |  |
| Analysis | 20 | In-depth analysis with strong reasoning and insights | Good analysis with logical reasoning | Basic analysis with limited depth | Weak or no analysis; lacks reasoning |  |
| Solution Design | 20 | Innovative, feasible solutions with strong justification | Logical solutions with adequate justification | Basic solutions with weak justification | Incorrect or no solution provided |  |
| Clarity | 10 | Clear, well-structured, and easy to understand | Organized and understandable presentation | Limited clarity and structure | Poor clarity; difficult to understand |  |

1. Preprocessing and Feature Detection

A vision system processes images captured under varying lighting conditions. Without preprocessing, feature detection results are inconsistent.Analyze the issue and explain how preprocessing improves feature detection performance.

2. Edge vs Corner Detection

In an image containing both straight edges and textured regions, a system uses edge detection but fails to identify key points for matching.Analyze why edge detection is insufficient and justify the use of corner detection in this case.

3. Hough Transform in Noisy Images

A system uses Hough Transform to detect lines in noisy images but produces inaccurate results.
Analyze the effect of noise on detection and suggest improvements based on preprocessing or parameter selection.

4. Feature Matching under Transformations

Two images of the same object are taken at different scales and orientations. Traditional feature matching fails, but scale invariant feature matching works effectively.Analyze why scale invariant methods perform better in this scenario.

5. PCA for Feature Optimization

A system extracts a large number of features, leading to high computational cost and redundancy.
Analyze how Principal Component Analysis helps in reducing dimensionality and improving efficiency.

1. Preprocessing and Feature Detection

Scenario

A vision system processes images captured under varying lighting conditions. Without preprocessing, feature detection results are inconsistent. Analyze the issue and explain how preprocessing improves feature detection performance.

Answer

In a computer vision system, images may be captured under different lighting conditions such as bright sunlight, low light, shadows, or uneven illumination. These changes affect the pixel intensity and contrast of the image. If feature detection is directly applied to these images without preprocessing, the same object may produce different features in different images.

For example, consider a camera observing the same object during the morning and evening. In the morning, the object may have sufficient illumination and its corners and edges may be clearly visible. In the evening, the same corners may become darker and less distinguishable. Consequently, the feature detector may detect different points.

Problems caused by varying lighting

- Change in pixel intensity – Brightness values change according to illumination.

- Reduced contrast – Important edges and corners may become difficult to identify.

- Shadows – Shadows can introduce unwanted edges and features.

- Noise – Low-light images often contain more noise.

- False features – Uneven illumination can create features that do not actually belong to the object.

- Inconsistent detection – The same feature may be detected in one image but not another.

Role of preprocessing

Preprocessing prepares the image before feature detection.

A typical workflow is:

Original Image

↓

Noise Removal

↓

Grayscale Conversion

↓

Contrast Enhancement

↓

Intensity Normalization

↓

Feature Detection

↓

Reliable Features

Important preprocessing techniques

1.Grayscaleconversion:
Converts a color image into a single intensity channel. This simplifies processing and reduces unnecessary information.

2.Noiseremoval:
Gaussian or median filtering can reduce unwanted noise while preserving important structures.

3.Histogramequalization:
Improves contrast by redistributing intensity values. It is useful when the image has poor contrast.

4.Adaptivehistogramequalization:
Techniques such as CLAHE can improve local contrast, which is useful for images containing uneven illumination.

5.Normalization:
Makes intensity values more consistent across different images.

How it improves feature detection

After preprocessing, important edges and corners become more visible. Noise and unwanted variations are reduced. Therefore, detectors such as Harris, Shi-Tomasi, SIFT, or ORB can identify features more consistently.

Example

Without preprocessing:

Dark Image → Weak edges/corners → Poor feature detection

With preprocessing:

Dark Image

↓

Contrast Enhancement + Noise Removal

↓

Clear edges/corners

↓

Better feature detection

Conclusion

Preprocessing is an important step in computer vision because it reduces the effects of illumination, noise, and contrast variations. It makes important image structures more distinguishable and improves the accuracy, reliability, and repeatability of feature detection.

2. Edge Detection vs Corner Detection

Scenario

An image contains both straight edges and textured regions. A system uses edge detection but fails to identify key points for matching. Analyze why edge detection is insufficient and justify the use of corner detection.

Answer

Edge detection and corner detection are both important techniques in computer vision, but they serve different purposes.

Edge detection identifies locations where there is a significant change in image intensity. It is useful for detecting object boundaries, lines, and shapes.

However, the scenario requires key points for feature matching. In this situation, edge detection alone is insufficient because points along a straight edge are often very similar.

Why edge detection fails

Consider a straight edge:

Edge

-------------------------

•  •  •  •  •  •  •  •

Every point along the edge has similar local information. If an image is shifted or transformed, it becomes difficult to determine exactly which point corresponds to a particular point in another image.

Therefore, edges provide boundary information, but they may not provide sufficiently distinctive key points.

Why corners are better

A corner is a point where image intensity changes significantly in two different directions.

Example:

|

|

|

|__________

↑

Corner

A corner provides much more distinctive information than a point on a straight edge.

Advantages of corner detection

Corner points are:

- Highly localized

- More distinctive

- Suitable for feature matching

- Useful for tracking

- Useful for image registration

- Useful for object recognition

- Useful for motion estimation

Common corner detection algorithms include:

- Harris Corner Detector

- Shi-Tomasi Corner Detector

- FAST

Harris Corner Detection

Harris detection examines how the intensity changes when a small window is moved in different directions.

The basic idea is:

Flat region → Small change in all directions

Edge        → Large change in one direction

Corner      → Large change in two directions

Therefore, corners can be distinguished from flat regions and edges.

Scenario application

Suppose a camera needs to match two images of the same building.

Edge detection may identify:

|       |

|       |

|_______|

But corner detection can identify the exact points where the building's boundaries meet. These points can then be matched between the two images.

Conclusion

Edge detection is useful for identifying boundaries, but it does not always provide distinctive key points. Corner detection is more appropriate when the goal is feature matching, because corners contain strong intensity variations in multiple directions and provide stable, localized features.

3. Hough Transform in Noisy Images

Scenario

A system uses Hough Transform to detect lines in noisy images but produces inaccurate results. Analyze the effect of noise and suggest improvements using preprocessing or parameter selection.

Answer

The Hough Transform is a popular technique for detecting geometric shapes such as lines and circles. It works by converting image points into a parameter space and identifying regions with a high number of votes.

For line detection, the equation can be represented as:

Each edge point votes for possible values of and . Strong peaks in the accumulator indicate possible lines.

Effect of noise

When an image contains noise, unwanted pixels may be detected as edges. These noisy pixels also participate in the voting process.

Therefore:

Noise

↓

False edge points

↓

Incorrect votes

↓

False peaks in accumulator

↓

Incorrect lines

This can produce:

- False lines

- Multiple unwanted lines

- Incorrect line positions

- Broken lines

- Weak or unstable detections

Preprocessing solution

Before applying Hough Transform, the image should be cleaned.

A suitable workflow is:

Input Image

↓

Noise Filtering

↓

Canny Edge Detection

↓

Hough Transform

↓

Parameter Thresholding

↓

Detected Lines

1. Gaussian filtering

Gaussian filtering smooths the image and reduces high-frequency noise.

It is useful when the noise is relatively smooth or random.

2. Median filtering

Median filtering is particularly useful for removing salt-and-pepper noise while preserving edges.

3. Canny edge detection

Canny can identify strong and meaningful edges before Hough Transform is applied.

This reduces the number of irrelevant points entering the Hough voting process.

Parameter selection

The parameters of the Hough Transform are also important.

Threshold:
Determines how many votes are required before a line is considered valid.

- Too low → many false lines

- Too high → real lines may be missed

Minimumlinelength:
Removing very short lines can eliminate noise-related detections.

Maximumlinegap:
Controls how much gap can exist between points belonging to the same line.

Example

If a road image contains lane markings:

Without preprocessing:

Noise → many false lines → inaccurate lane detection

With preprocessing:

Noise removal → Canny → Hough → correct lane lines

Conclusion

Noise directly affects the accuracy of Hough Transform by generating false edge points and votes. The problem can be reduced by applying Gaussian/median filtering, Canny edge detection, and suitable Hough parameters. Proper preprocessing and parameter tuning result in more accurate and reliable line detection.

4. Feature Matching Under Transformations

Scenario

Two images of the same object are taken at different scales and orientations. Traditional feature matching fails, but scale-invariant feature matching works effectively. Analyze why scale-invariant methods perform better.

Answer

Feature matching attempts to identify corresponding points between two images. Traditional methods may work well when the images have similar size and orientation. However, their performance decreases when the object undergoes transformations such as scaling and rotation.

Problem with traditional matching

Suppose the first image contains a small object:

□

The second image contains the same object at a larger scale:

□

/ \

The pixel patterns around the feature are different.

Similarly, if the object is rotated, the orientation of local structures changes.

Therefore:

Scale change

↓

Feature size changes

Rotation

↓

Feature orientation changes

Both

↓

Traditional matching may fail

Scale-invariant feature matching

Scale-invariant techniques are designed to identify features even when their size changes.

One important example is SIFT (Scale-Invariant Feature Transform).

SIFT generally performs the following steps:

Input Image

↓

Scale-space construction

↓

Keypoint detection

↓

Keypoint localization

↓

Orientation assignment

↓

Feature descriptor generation

↓

Feature matching

Why scale invariance helps

Features are detected across multiple scales. Therefore, the same object feature can be detected whether the object appears:

- Small

- Large

- Near

- Far

Why rotation handling helps

The feature is assigned an orientation based on its local image structure. The descriptor is then constructed relative to that orientation.

Therefore, the descriptor remains more consistent even if the object is rotated.

Example

Suppose an image contains a logo.

Image 1:

Logo

↓

Normal size

Normal orientation

Image 2:

Logo

↓

Larger size

Rotated 45°

Traditional pixel-based matching may fail because the pixel arrangement is different.

A scale- and rotation-invariant descriptor can still identify corresponding keypoints.

Applications

Such methods are useful in:

- Object recognition

- Panorama creation

- Image registration

- Augmented reality

- Robot navigation

- Image retrieval

- Tracking

Conclusion

Scale-invariant feature matching performs better because it is designed to handle changes in scale and orientation. Instead of directly comparing raw pixels, it uses robust local feature descriptors, allowing the same object to be recognized under different viewing conditions.

5. PCA for Feature Optimization

Scenario

A system extracts a large number of features, leading to high computational cost and redundancy. Analyze how PCA reduces dimensionality and improves efficiency.

Answer

In computer vision and machine learning, feature extraction may produce hundreds or thousands of features. However, not all features are equally important. Some features may contain similar information, resulting in redundancy.

For example, a system may extract:

1000 Features

↓

Many correlated/redundant features

↓

High computation

↓

Slow processing

Problems with high-dimensional features

A large number of features can cause:

- High computational cost

- Increased memory usage

- Longer processing time

- Redundant information

- Difficulty in visualization

- Possible overfitting in some machine-learning tasks

PCA solution

Principal Component Analysis (PCA) is a dimensionality-reduction technique.

It transforms the original features into a new set of variables called principal components.

The components are arranged according to the amount of variance they capture.

The basic workflow is:

Original Dataset

↓

Feature Standardization

↓

Calculate Covariance

↓

Find Principal Components

↓

Select Important Components

↓

Reduced Feature Set

Example

Suppose the system generates:

1000 original features

After PCA, it may retain:

100 principal components

1000 Features

↓

PCA

↓

100 Important Components

Instead of processing all 1000 features, the machine-learning system processes only 100 components.

How PCA selects features

PCA looks for directions in the data that contain the greatest variance.

The first principal component contains the largest amount of variance.

The second component contains the next largest amount, and so on.

Less informative components can be discarded.

Benefits

1.Reducedcomputationalcost
Fewer dimensions mean fewer calculations.

2.Reducedmemoryusage
The system needs to store fewer values.

3.Reducedredundancy
Highly correlated information is represented more compactly.

4.Fasterprocessing
Classification and prediction can become faster.

5.Easiervisualization
High-dimensional data can be represented using two or three principal components.

Scenario application

Suppose a facial recognition system extracts 2,000 features from each face.

Processing all 2,000 features for thousands of faces is expensive.

PCA can transform them into a smaller representation:

2000 Face Features

↓

PCA

↓

200 Important Components

↓

Face Recognition

The system can therefore process the data more efficiently while retaining most of the important variation.

Important point

PCA does not simply select the original features. It creates new features called principal components, which are combinations of the original features.

Conclusion

PCA reduces the dimensionality of a feature set while preserving the most significant information. This reduces redundancy, memory usage, and computational cost, making the computer vision system faster and more efficient.